| General information |  |
| --- | --- |
| Orderer / Customer | Famar Group, Technologiepark 5, 66424 Homburg, Germany |
| Inspection provider | IMP Promont d.o.o., Pot k sejmišču 30a, 1231 Črnuče |
| Client requirements | Videoscopic and visual inspection of welds on stainless steel pipelines |
| Acceptance criterion | Fraudulence of the weld root, root protection, weld quality B according to SIST EN ISO 5817, level of coloration of the inside of the pipeline 3 according to AWS D18.2. |
| Place of inspection | IMP Promont PE Naklo, Polica 42, 4202 Naklo |
| Extent of examination | 100% weld inspection |

| Subject of review |  |
| --- | --- |
| The object of inspection | {{Sklop}} |
| Basic material | AISI 316L |
| Filler material | / |
| Welder | Matej Lichon |
| Welding process | TIG – orbital welding |
| Weld tipe | " I " |
| No. of examined welds | {{SteviloZvarov}} welds |

Videoscopic and visual inspection standards

SIST EN ISO 17637: Non-destructive testing of welds - Visual testing of fusion-welded joints

SIST EN 13018: Non-destructive testing - Visual testing – General principles

SIST EN ISO 6520-1: Classification of geometric imperfections in metallic materials-Part 1: Fusion welding

SIST ISO 3058: Non-destructive testing — Aids to visual inspection — Selection of low-power magnifiers

AWS D18.1/D18.1M: Specification for Welding of Austenitic Stainless Steel Tube and Pipe Systems in Sanitary (Hygienic) Applications

SIST EN ISO 5817: Welding - Fusion-welded joints in steel, nickel, titanium and their alloys (beam welding excluded) - Quality levels for imperfections

| Equipment used |  |
| --- | --- |
| Industrial videoscope | Olympus Iplex SA – IV94435G; Tov. št. Y404961 |
| Optical attachment | AT120S/NF – IV96 |
| Optical attachment | AT120D/FF – IV96 |
| Flashlight | Mini maglite |
| Mirror with telescopic handle | Midlock A-2-T |

A direct and indirect visual examination was performed in accordance with SIST EN 13018 and SIST EN ISO 17637.

Findings of direct visual examination

The welded joints examined by direct examination (the face side of the welds) are made without major peculiarities and do not show deviations from the prescribed permitted geometric imperfections.

Findings of the videoscopic examination

The inspected welds are free from unacceptable defects and are acceptable. The pipe will need to be passivated before use. See annex 01.

Weld locations are marked on:

- Isometry no. {{Isometrija}}

| EXAMINATION DONE BY: Gašper Žerjav, VT level II. {{Datum}} |  | EVALUATION DONE BY: Gašper Žerjav, VT level II. {{Datum}} |
| --- | --- | --- |
| Name and surname Date / signature |  | Name and surname Date / signature |

Annex 01:

- Isometry no. {{Isometrija}}

{{Fotografije}}